General Information

The physics behind an aircraft

To explain how an aircraft flies you should start by thinking of the forces. There are 4 forces on an aircraft. The first one is applied to everything on earth, this one is called the weight force or gravity. This creates a downwards force. We can calculate the downwards force with the formula m X g. M being the mass and g being gravity which on earth is about 9.81. But of course an aircraft moves up. For this we have a force called lift. Lift operates perpendicular to the aircrafts wings. The third force is called thrust, this force makes the aircraft move forwards. Thrust on an airplane can be created by a propeller or a jet engine. The principal of such an engine is that it accelerates air out of the back and then follows newton’s third law. This can be proven by a balloon filled with air an opening it. The balloon will shoot forward. The last force on an aircraft is drag. It moves the aircraft backwards. Drag is created mostly by air resistance. We can minimalize this drag by making the aircraft aerodynamic. This means that it has smooth lines.

We can calculate the lift with the equation of magnitude of lift per unit wing area. This equation is L=0.5 X P X Cl X V squared. In this formula L is of course lift. P is the density of the air which creates the resistance. This resistance is normally 1.2754 kg/m cubed. Cl is the coefficient of the lift. This coefficient has to do with the wings and changes with the angle of attack. The angle of attack is the angle between the nose of the aircraft and the horizontal line formed underneath the wings. This horizontal line is formed because even if an airplane is taking of it is still partially moving horizontally. As you increase the angle of attack the lift rises until a certain point. At this point the wings are almost vertically and create a surface of drag force. This point is called stall. Here is a graph of the coefficient of lift.

There is a similar equation for the drag on an airplane. This equation is 
D=0.5 X P X Cd X V squared. This formula works the same accept for that Cd is the coefficient of drag. This also has to do with the angle of attack. As the angle of attack increases the coefficient of drag also increases. This is because as the aircraft is pitching there is more wing area perpendicular to the flow. Here is a graph of the Cd.

Facts about aircrafts

About 4.5 million people fly every day.

About 16.5 billion people fly on a yearly basis.

Tupolev TU 144 is the fastest passenger aeroplane ever built.

The maximum capacity of an A380 is 853 passengers.

The A380 is also the most fuel efficient aeroplane per capita.

Around 3 billion people boarded some 35 million flights last year.

Only 224 people died in an air crash last year.

In fact, 2013 was the safest year ever for airlines, surpassing 2012 (which was the safest ever), which surpassed 2011 (which was the safest ever)–you get the picture.