General Information

The physics behind an aircraft

To explain how an aircraft flies you should start by thinking of the forces. There are 4 forces on an aircraft. The first one is applied to everything on earth, this one is called the weight force or gravity. This creates a downwards force. We can calculate the downwards force with the formula m * g. M being the mass and g being gravity which on earth is about 9.81. But of course an aircraft moves up. For this we have a force called lift. Lift operates perpendicular to the aircrafts wings. The third force is called thrust, this force makes the aircraft move forwards. Thrust on an aeroplane can be created by a propeller or a jet engine. The principal of such an engine is that it accelerates air out of the back and then follows newton’s third law. This can be proven by a balloon filled with air an opening it. The balloon will shoot forward. The last force on an aircraft is drag. It moves the aircraft backwards. Drag is created mostly by air resistance. We can minimalize this drag by making the aircraft aerodynamic. This means that it has smooth lines.

We can calculate the lift with the equation of magnitude of lift per unit wing area. This equation is L=0.5 * P * Cl * V squared. In this formula L is of course lift. P is the density of the air which creates the resistance. This resistance is normally 1.2754 kg/m cubed. Cl is the coefficient of the lift. This coefficient has to do with the wings and changes with the angle of attack. The angle of attack is the angle between the nose of the aircraft and the horizontal line formed underneath the wings. This horizontal line is formed because even if an aeroplane is taking of it is still partially moving horizontally. As you increase the angle of attack the lift rises until a certain point. At this point the wings are almost vertically and create a surface of drag force. This point is called stall. Here is a graph of the coefficient of lift.

There is a similar equation for the drag on an aeroplane. This equation is 
D=0.5 * P * Cd * V squared. This formula works the same accept for that Cd is the coefficient of drag. This also has to do with the angle of attack. As the angle of attack increases the coefficient of drag also increases. This is because as the aircraft is pitching there is more wing area perpendicular to the flow. Here is a graph of the Cd.

Facts about aircrafts

Air transportation is the safest form of travelling in the world.

A plane lands somewhere in the world every 3 seconds.

If the chance of daily disaster was only 0.01%, this would mean that 13 air-planes must crash. The odd of a plane crash are around 0.001%.

Donkeys kill more people annually than plane crashes.

A person is 10 times more likely to be killed by a car while standing at a crosswalk waiting for the green light.

Before each take-off, the plane goes through complex technical inspection.

Plane crashes are never a coincidence, but always a combination of highly unusual faults inside a plane's infrastructure.

More than 80% of the population human is afraid of flying. 5% completely abandons flying and takes alternative forms of travel. The scientific name of fear and flight altitude - aerophobia.

More people fall in love with flight attendants than representatives of other professions.

Air pumped in the planes is one of the cleanest in the world.