Acronym: A word constructed by taking the initial letters of a string of words. For example: RAM is an acronym for Random Access Memory, and CPU is an acronym for Central Processing Unit.

Active matrix display: A liquid crystal display using a transistor to control the transmission of light at each individual pixel.

Address: A value used to delineate the location of a specifi c data element within a memory array.

Alignment restriction: A requirement that data be aligned in memory on natural boundaries.

Amdahl’s Law: A rule stating that the performance enhancement possible with a given improvement is limited by the amount that the improved feature is used. It is a quantitative version of the law of diminishing returns.

AND: A logical bit-by-bit operation with two operands that calculates a 1 only if there is a 1 in both operands.

Antidependence: Also called name dependence. An ordering forced by the reuse of a name, typically a register, rather than by a true dependence that carries a value between two instructions.

Application binary interface (ABI): The user portion of the instruction set plus the operating system interfaces used by application programmers. It defines a standard for binary portability across computers.

Architectural registers: The instruction set of visible registers of a processor; for example, in MIPS, these are the 32 integer and 16 floatingpoint registers.

Arithmetic Logic Unit (ALU): Hardware that performs addition, subtraction, and usually logical operations such as AND and OR.

Assembler: A program that translates a symbolic version of instructions into the binary version.

Assembly language: A symbolic language that can be translated into binary machine language.

Assembly language: A symbolic representation of machine instructions.

Asserted: The signal is logically high or true.

Benchmark: A program selected for use in comparing computer performance.

Biased notation: A notation that represents the most negative value by 00... 000 two and the most positive value by 11... 11 two, with 0 typically having the value 10... 00 two, thereby biasing the number such that the number plus the bias has a non-negative representation.

Binary digit: Also called a bit. One of the two numbers in base 2 (0 or 1) that are the components of information.

Binary digit: Also called binary bit. One of the two numbers in base 2, 0 or 1, that are the components of information.

Branch delay slot: The slot directly aft er a delayed branch instruction, which in the MIPS architecture is filled by an instruction that does not affect the branch.

Branch not taken or (untaken branch): A branch where the branch condition is false and the program counter (PC) becomes the address of the instruction that sequentially follows the branch.

Branch prediction buffer: Also called branch history table. A small memory that is indexed by the lower portion of the address of the branch instruction and that contains one or more bits indicating whether the branch was recently taken or not.

Branch prediction: A method of resolving a branch hazard that assumes a given outcome for the branch and proceeds from that assumption rather than waiting to ascertain the actual outcome.

Branch taken: A branch where the branch condition is satisfied and the program counter (PC) becomes the branch target. All unconditional jumps are taken branches.

Branch target address: The address specified in a branch, which becomes the new program counter (PC) if the branch is taken. In the MIPS architecture the branch target is given by the sum of the off set field of the instruction and the address of the instruction following the branch.

Branch target buffer: A structure that caches the destination PC or destination instruction for a branch. It is usually organized as a cache with tags, making it more costly than a simple prediction buffer.

Branch: A type of branch where the instruction immediately following the branch is always executed, independent of whether the branch condition is true or false.

Cache memory: A small, fast memory that acts as a buff er for a slower, larger memory.

Callee: A procedure that executes a series of stored instructions based on parameters provided by the caller and then returns control to the caller.

Caller: The program that instigates a procedure and provides the necessary parameter values.

Central processor unit (CPU): Also called processor. The active part of the computer, which contains the datapath and control and which adds numbers, tests numbers, signals I/O devices to activate, and so on.

Clock cycle: Also called tick, clock tick, clock period, clock, or cycle. The time for one clock period, usually of the processor clock, which runs at a constant rate.

Clock cycles per instruction (CPI): Average number of clock cycles per instruction for a program or program fragment.

Clock period: The length of each clock cycle.

Clocking methodology: The approach used to determine when data is valid and stable relative to the clock.

Cloud Computing: refers to large collections of servers that provide services over the Internet; some providers rent dynamically varying numbers of servers as a utility.

Combinational element: An operational element, such as an AND gate or an ALU.

Commit unit: The unit in a dynamic or out-of-order execution pipeline that decides when it is safe to release the result of an operation to programmervisible registers and memory.

Compiler: A program that translates high-level language statements into assembly language statements.

Conditional branch: An instruction that requires the comparison of two values and that allows for a subsequent transfer of control to a new address in the program based on the outcome of the comparison.

Control hazard: Also called branch hazard. When the proper instruction cannot execute in the proper pipeline clock cycle because the instruction that was fetched is not the one that is needed; that is, the flow of instruction addresses is not what the pipeline expected.

Control signal: A signal used for multiplexor selection or for directing the operation of a functional unit; contrasts with a data signal, which contains information that is operated on by a functional unit.

Control: The component of the processor that commands the datapath, memory, and I/O devices according to the instructions of the program.

Correlating predictor: A branch predictor that combines local behavior of a particular branch and global information about the behavior of some recent number of executed branches.

CPU execution time: Also called CPU time. The actual time the CPU spends computing for a specific task.

Data hazard: Also called a pipeline data hazard. When a planned instruction cannot execute in the proper clock cycle because data that is needed to execute the instruction is not yet available.

Data race: Two memory accesses form a data race if they are from diff erent threads to same location, at least one is a write, and they occur one aft er another.

Data transfer instruction: A command that moves data between memory and registers.

Datapath element: A unit used to operate on or hold data within a processor. In the MIPS implementation, the datapaThelements include the instruction and data memories, the register file, the ALU, and adders.

Datapath: The component of the processor that performs arithmetic operations

Deasserted: The signal is logically low or false.

Defect: A microscopic flaw in a wafer or in patterning steps that can result in the failure of the die containing that defect.

Die: The individual rectangular sections that are cut from a wafer, more informally known as chips.

Dividend: A number being divided.

Divisor: A number that the dividend is divided by.

Don’t-care term: An element of a logical function in which the output does not depend on the values of all the inputs. Don’t-care terms may be specifi ed in different ways.

Double precision: A floating-point value represented in two 32-bit words.

Dynamic branch prediction: Prediction of branches at runtime using runtime information.

Dynamic multiple issue: An approach to implementing a multipleissue processor where many decisions are made during execution by the processor.

Dynamic pipeline scheduling: Hardware support for reordering the order of instruction execution so as to avoid stalls.

Dynamic random access memory (DRAM): Memory built as an integrated circuit; it provides random access to any location. Access times are 50 nanoseconds and cost per gigabyte in 2012 was $5 to $10.

Dynamically linked libraries (DLLs): Library routines that are linked to a program during execution.

Edge-triggered clocking: A clocking scheme in which all state changes occur on a clock edge.

Embedded computer: A computer inside another device used for running one predetermined application or collection of software.

Exception: Also called interrupt. An unscheduled event that disrupts program execution; used to detect overflow.

Executable file: A functional program in the format of an object file that contains no unresolved references. It can contain symbol tables and debugging information. A “stripped executable” does not contain that information. Relocation information may be included for the loader.

Exponent: In the numerical representation system of floating-point arithmetic, the value that is placed in the exponent field.

Flash memory: A nonvolatile semiconductor memory. It is cheaper and slower than DRAM but more expensive per bit and faster than magnetic disks. Access times are about 5 to 50 microseconds and cost per gigabyte in 2012 was $0.75 to $1.00.

Floating point: Computer arithmetic that represents numbers in which the binary point is not fixed.

Flush: To discard instructions in a pipeline, usually due to an

Forwarding: Also called bypassing. A method of resolving a data hazard by retrieving the missing data element from internal buffers rather than waiting for it to arrive from programmervisible registers or memory.

Fraction: The value, generally between 0 and 1, placed in the fraction field. The fraction is also called the mantissa.

Frame pointer: A value denoting the location of the saved registers and local variables for a given procedure.

High-level programming language: A portable language such as C, C++, Java, or Visual Basic that is composed of words and algebraic notation that can be translated by a compiler into assembly language.

Implementation: Hardware that obeys the architecture abstraction.

Imprecise interrupt: Also called imprecise exception. Interrupts or exceptions in pipelined computers that are not associated with the exact instruction that was the cause of the interrupt or exception.

In-order commit: A commit in which the results of pipelined execution are written to the programmer visible state in the same order that instructions are fetched.

Input device: A mechanism through which the computer is fed information, such as a keyboard.

Instruction count: the number of instructions executed by the program.

Instruction format: A form of representation of an instruction composed of fields of binary numbers.

Instruction latency: The inherent execution time for an instruction.

instruction mix: A measure of the dynamic frequency of instructions across one or many programs.

Instruction set architecture: Also called architecture. An abstract interface between the hardware and the lowest-level soft ware that encompasses all the information necessary to write a machine language program that will run correctly, including instructions, registers, memory access, I/O, and so on.

Instruction set: The vocabulary of commands understood by a given architecture.

Instruction-level parallelism: The parallelism among instructions.

Instruction: A command that computer hardware understands and obeys.

Integrated circuit: Also called a chip. A device combining dozens to millions of transistors.

Interrupt: An exception that comes from outside of the processor. (Some architectures use the term interrupt

Interrupt: An exception that comes from outside of the processor. (Some architectures use the term interrupt for all exceptions.)

Issue packet: The set Static Multiple Issue of instructions that issues together in one clock cycle; the packet may be determined statically by the compiler or dynamically by the processor.

Issue slots: The positions from which instructions could issue in a given clock cycle; by analogy, these correspond to positions at the starting blocks for a sprint.

Java bytecode: Instruction from an instruction set designed to interpret Java programs.

Java Virtual Machine (JVM): The program that interprets Java bytecodes.

Jump address table: Also called jump table. A table of addresses of alternative instruction sequences.

Jump-and-link instruction: An instruction that jumps to an address and simultaneously saves the address of the following instruction in a register ($ra in MIPS).

Just In Time compiler (JIT): The name commonly given to a compiler that operates at runtime, translating the interpreted code segments into the native code of the computer.

Latency (pipeline): The number of stages in a pipeline or the number of stages between two instructions during execution.

Least significant bit: The rightmost bit in a MIPS word.

Linker: Also called link editor. A systems program that combines independently assembled machine language programs and resolves all undefined labels into an executable file.

Liquid crystal display: A display technology using a thin layer of liquid polymers that can be used to transmit or block light according to whether a charge is applied.

Load-use data hazard: A specific form of data hazard in which the data being loaded by a load instruction has not yet become available when it is needed by another instruction.

Loader: A system’s program that places an object program in main memory so that it is ready to execute.

Local area network (LAN): A network designed to carry data within a geographically confined area, typically within a single building.

Loop unrolling: A technique to get more performance from loops that access arrays, in which multiple copies of the loop body are made and instructions from different iterations are scheduled together

Machine language: A binary representation of machine instructions.

Machine language: Binary representation used for communication within a computer system. hexadecimal Numbers in base 16.

Magnetic disk: Also called hard disk. A form of nonvolatile secondary memory composed of rotating platters coated with a magnetic recording material. Because they are rotating mechanical devices, access times are about 5 to 20 milliseconds and cost per gigabyte in 2012 was $0.05 to $0.10.

Main memory: Also called primary memory. Memory used to hold programs while they are running; typically consists of DRAM in today’s computers.

Memory: The storage area in which programs are kept when they are running and that contains the data needed by the running programs.

Micro-architecture: The organization of the processor, including the major functional units, their interconnection, and control.

Million instructions per second (MIPS): A measurement of program execution speed based on the number of millions of instructions. MIPS is computed as the instruction count divided by the product of the execution time and 106.

Most significant bit: The left most bit in a MIPS word.

Multicore microprocessor: A microprocessor containing multiple processors (“cores”) in a single integrated circuit.

Multiple issue: A scheme whereby multiple instructions are launched in one clock cycle.

Nonvolatile memory: A form of memory that retains data even in the absence of a power source and that is used to store programs between runs. A DVD disk is nonvolatile.

Nop: An instruction that does no operation to change state.

NOR: A logical bit-by-bit operation with two operands that calculates the NOT of the OR of the two operands. Th at is, it calculates a 1 only if there is a 0 in both operands.

Normalized: A number in floating-point notation that has no leading 0s.

NOT: A logical bit-by-bit operation with one operand that inverts the bits; that is, it replaces every 1 with a 0, and every 0 with a 1.

Object oriented language: A programming language that is oriented around objects rather than actions, or data versus logic.

One’s complement: A notation that represents the most negative value by 10... 000 two and the most positive value by 01... 11 two, leaving an equal number of negatives and positives but ending up with two zeros, one positive (00... 00 two ) and one negative (11... 11 two ). The term is also used to mean the inversion of every bit in a pattern: 0 to 1 and 1 to 0.

Opcode: The field that denotes the operation and format of an instruction.

OR: A logical bit-by-bit operation with two operands that calculates a 1 if there is a 1 in either operand.

Out-of-order execution: A situation in pipelined execution when an instruction blocked from executing does not cause the following instructions to wait.

Output device: A mechanism that conveys the result of a computation to a user, such as a display, or to another computer.

Overflow (floating point): A situation in which a positive exponent becomes too large to fit in the exponent field.

PC-relative addressing: An addressing regime in which the address is the sum of the program counter (PC) and a constant in the instruction.

Personal computer (PC): A computer designed for use by an individual, usually incorporating a graphics display, a keyboard, and a mouse.

Personal mobile devices (PMDs): are small wireless devices to connect to the Internet; they rely on batteries for power, and software is installed by downloading apps. Conventional examples are smart phones and tablets.

Pipeline stall: Also called bubble. A stall initiated in order to resolve a hazard.

Pipelining: An implementation technique in which multiple instructions are overlapped in execution, much like an assembly line.

Pixel: The smallest individual picture element. Screens are composed of hundreds of thousands to millions of pixels, organized in a matrix.

Pop: Remove element from stack.

Precise interrupt: Also called precise exception. An interrupt or exception that is always associated with the correct instruction in pipelined computers.

Procedure frame: Also called activation record. The segment of the stack containing a procedure’s saved registers and local variables.

Procedure: A stored subroutine that performs a specific task based on the parameters with which it is provided.

Program counter (PC): The register containing the address of the instruction in the program being executed.

Pseudo-instruction: A common variation of assembly language instructions oft en treated as if it were an instruction in its own right.

Push: Add element to stack.

Quotient: The primary result of a division; a number that when multiplied by the divisor and added to the remainder produces the dividend.

Register file: A state element that consists of a set of registers that can be read and written by supplying a register number to be accessed.

Register renaming: The renaming of registers by the compiler or hardware to remove anti-dependences.

Remainder: The secondary result of a division; a number that when added to the product of the quotient and the divisor produces the dividend.

Reorder buffer: The buffer that holds results in a dynamically scheduled processor until it is safe to store the results to memory or a register.

Reservation station: A buffer within a functional unit that holds the operands and the operation.

Response time: Also called execution time. The total time required for the computer to complete a task, including disk accesses, memory accesses, I/O activities, operating system overhead, CPU execution time, and so on.

Return address: A link to the calling site that allows a procedure to return to the proper address; in MIPS it is stored in register $ra.

Scientific notation: A notation that renders numbers with a single digit to the left of the decimal point.

Secondary memory: Nonvolatile memory used to store programs and data between runs; typically consists of fl ash memory in PMDs and magnetic disks in servers.

Server: A computer used for running larger programs for multiple users, often simultaneously, and typically accessed only via a network.

Sign-extend: To increase the size of a data item by replicating the high-order sign bit of the original data item in the high order bits of the larger, destination data item.

silicon crystal ingot: A rod composed of a silicon crystal that is between 8 and 12 inches in diameter and about 12 to 24 inches long.

Silicon: A natural element that is a semiconductor.

Single precision: A floating-point value represented in a single 32bit word.

Single-cycle implementation: Also called single clock cycle implementation. An implementation in which an instruction is executed in one clock cycle. While easy to understand, it is too slow to be practical.

Soft ware as a Service (SaaS): delivers software and data as a service over the Internet, usually via a thin program such as a browser that runs on local client devices, instead of binary code that must be installed, and runs wholly on that device. Examples include web search and social networking.

Speculation: An approach whereby the compiler or processor guesses the outcome of an instruction to remove it as a dependence in executing other instructions.

Stack pointer: A value denoting the most recently allocated address in a stack that shows where registers should be spilled or where old register values can be found. In MIPS, it is register $sp.

Stack: A data structure for spilling registers organized as a last-infirst-out queue.

State element: A memory element, such as a register or a memory.

Static multiple issue: An approach to implementing a multiple-issue processor where many decisions are made by the compiler before execution.

Static random access memory (SRAM): Also memory built as an integrated circuit, but faster and less dense than DRAM.

Stored-program concept: The idea that instructions and data of many types can be stored in memory as numbers, leading to the stored-program computer.

Structural hazard: When a planned instruction cannot execute in the proper clock cycle because the hardware does not support the combination of instructions that are set to execute.

Supercomputer: A class of computers with the highest performance and cost; they are configured as servers and typically cost tens to hundreds of millions of dollars.

Superscalar: An advanced pipelining technique that enables the processor to execute more than one instruction per clock cycle by selecting them during execution.

Symbol table: A table that matches names of labels to the addresses of the memory words that instructions occupy.

System CPU time: The CPU time spent in the operating system performing tasks on behalf of the program.

Terabyte (TB): Originally 1,099,511,627,776 (240) bytes, although communications and secondary storage systems developers started using the term to mean 1,000,000,000,000 (1012) bytes. To reduce confusion, we now use the term tebibyte (TiB) for 240 bytes, defining terabyte (TB) to mean 1012 bytes.

Text segment: The segment of a UNIX object file that contains the machine language code for routines in the source file.

Throughput: Also called bandwidth. Another measure of performance, it is the number of tasks completed per unit time.

Tournament branch predictor: A branch predictor with multiple predictions for each branch and a selection mechanism that chooses which predictor to enable for a given branch.

Transistor: An on/off switch controlled by an electric signal.

Truth table: From logic, a representation of a logical operation by listing all the values of the inputs and then in each case showing what the resulting outputs should be.

Underflow (floating point): A situation in which a negative exponent becomes too large to fit in the exponent field.

unexpected event.

Use latency: Number of clock cycles between a load instruction and an instruction that can use the result of the load without stalling the pipeline.

User CPU time: The CPU time spent in a program itself.

vectored interrupt An interrupt for which the address to which control is transferred is determined by the cause of the exception.

Very large-scale integrated (VLSI) circuit: A device containing hundreds of thousands to millions of transistors.

Very Long Instruction Word (VLIW): A style of instruction set architecture that launches many operations that are defined to be independent in a single wide instruction, typically with many separate opcode fields.

Volatile memory: Storage, such as DRAM, that retains data only if it is receiving power.

Wafer: A slice from a silicon ingot no more than 0.1 inches thick, used to create chips.

Wide area network (WAN): A network extended over hundreds of kilometers that can span a continent.

Word: The natural unit of access in a computer, usually a group of 32 bits; corresponds to the size of a register in the MIPS architecture.

Workload: A set of programs run on a computer that is either the actual collection of applications run by a user or constructed from real programs to approximate such a mix. A typical workload specifies both the programs and the relative frequencies.

Yield: The percentage of good dies from the total number of dies on the wafer.